Sheen Zar Agriculture Trading Company

REQUEST FOR QUOTATION (RFQ)
Procurement, Delivery and Installation of Concrete I-Trellis Kits for High-Density Vineyards and Orchards

RFQ Reference Number: SZATC/2025/RFQ/001

### TABLE OF CONTENTS

- Provisional Timetable

- Introduction

- Purpose of this RFQ

- Submission of Quotations

- Mandatory Requirements

- Evaluation and Award Criteria

- RFQ Queries and Clarifications

- Conflict of Interest

- Payment Terms

- Quantity Requirements

### 1. PROVISIONAL TIMETABLE

| Activity | Date |
| --- | --- |
| RFQ Issuance | 22 April 2025 |
| Deadline for Supplier Queries | 24 April 2025 |
| Responses to Queries | 25 April 2025 |
| Quotation Submission Deadline | 30 April 2025 |
| Quotation Opening and Evaluation | 1 May 2025 |
| Contract Award | 5 May 2025 |
| Contract Start Date | 10 May 2025 |

### 2. INTRODUCTION

Sheen Zar Agriculture Trading Company invites qualified local concrete producers and construction companies registered in Afghanistan to submit quotations for supplying concrete I-Trellis Kits for high-density vineyards and orchards.

Priority will be given to local companies based in respective provinces or zones.

### 3. PURPOSE OF THIS RFQ

To procure concrete I-Trellis kits conforming strictly to technical specifications provided, ensuring timely delivery and installation in specified districts and provinces across Afghanistan.

### 3.1. TECHNICAL SPECIFICATIONS (I-Trellis Kit)

| No. | Item | Description | Unit | Quantity per Jerib |
| --- | --- | --- | --- | --- |
| 1 | Concrete trellis post | Concrete trellis post (4 metal wires, 8 mm diameter, with 8 hooks), 10 cm diameter x 3.7 m length with five holes, first hole 30 cm above soil surface, top hole 10 cm down from top, Concrete M20(1:1.5:3), gravel max. 12.5 mm, spacing between holes 65 cm for 3 mm wire, 17 rows, 8 posts per row. | Piece | 136 |
| 2 | Hot galvanized wire | Hot galvanized wire 3 mm diameter | Kg | 150 |
| 3 | End T-bar | End T-bar rod 50 cm diameter, 50 cm length, 3 wires 5 mm diameter, 3 hooks, center hook 16 mm iron, Concrete M20(1:1.5:3), gravel max. 12.5 mm | Piece | 34 |
| 4 | Gripple | Gripple wire joiner | Piece | 170 |
| ***Total 197 Jerib of land x 136 Trellis = 26792 |  |  |  |  |

### 4. SUBMISSION OF QUOTATIONS

- All documents must be signed and stamped.

- Quotations must be submitted via email: quote@sheenzar.com

Subject line = "Quotation for RFQ Number: SZATC/2025/RFQ/001"

Deadline: On or before 11:00 AM, 30 April 2025

Late submissions will not be considered.

### 5. MANDATORY REQUIREMENTS

Bidders must submit:

- Valid Company Registration

- Tax Identification Document

Non-submission of these mandatory requirements will result in exclusion.

### 6. EVALUATION AND AWARD CRITERIA

- Preference given to suppliers based on provincial or zonal proximity.

- Evaluation will be based on:

| Criteria | Weight (%) |
| --- | --- |
| Price Competitiveness | 60% |
| Technical Capability & Experience | 20% |
| Delivery Time | 10% |
| Local Presence/Province | 10% |

### 7. RFQ QUERIES AND CLARIFICATIONS

Queries should be emailed by 30 April 2025 to:

quote@sheenzar.com

### 8. CONFLICT OF INTEREST

Suppliers must declare any potential or actual conflicts of interest. Failure to disclose conflicts may lead to disqualification.

### 9. PAYMENT TERMS

- Payments through bank transfer within 21 days of successful delivery.

- Quotations must be in AFN. Exchange rate reference: DA Afghanistan Bank.

### 10. QUANTITY REQUIREMENTS

| Zone | Province | District | Trellis Qty | Unit Price | Total Price |
| --- | --- | --- | --- | --- | --- |
| Central | Kabul | Charasyab | 952 |  |  |
| Central | Maidan Wardak | Sayedabad | 952 |  |  |
| Central | Maidan Wardak | Jalriz | 680 |  |  |
| Central Highland | Bamyan | Center | 952 |  |  |
| Central Highland | Bamyan | Yakawolang | 952 |  |  |
| Central Highland | Daikundi | Provincial Capital (Nili) | 1360 |  |  |
| Central Highland | Daikundi | Shahristan | 1224 |  |  |
| Sub-Total ZONE 1 |  |  |  |  |  |
| East | Kunar | Khas Kunar | 952 |  |  |
| East | Kunar | Asadabad | 952 |  |  |
| East | Laghman | Qarghaee | 952 |  |  |
| East | Nangarhar | Khugiani | 952 |  |  |
| East | Nuristan | Bargimatal | 1224 |  |  |
| Sub-Total ZONE 2 |  |  |  |  |  |
| North | Balkh | Balkh | 952 |  |  |
| North | Samangan | Aybak | 952 |  |  |
| North East | Badakhshan | Ishkashim | 952 |  |  |
| North East | Baghlan | Khost Wa Fering | 952 |  |  |
| North East | Baghlan | Andarab | 952 |  |  |
| North East | Takhar | Farkhar | 952 |  |  |
| Sub-Total ZONE 3 |  |  |  |  |  |
| South East | Ghazni | Ghazni center | 952 |  |  |
| South East | Ghazni | Andar | 952 |  |  |
| South East | Khost | Nadersha Kot | 680 |  |  |
| South East | Khost | Musakhil | 680 |  |  |
| South East | Paktika | Ziruk | 952 |  |  |
| South East | Paktika | Mata Khan | 952 |  |  |
| South East | Paktiya | Rohani Baba | 952 |  |  |
| South East | Paktiya | Gardiz | 952 |  |  |
| Sub-Total ZONE 4 |  |  |  |  |  |
| West | Ghor | Firoozkoh | 952 |  |  |
| West | Ghor | Saghar | 952 |  |  |
| Sub-Total ZONE 5 |  |  |  |  |  |
| TOAL |  |  |  |  |  |

Authorized by:
Sheen Zar Agriculture Trading Co.
Kabul, Afghanistan
22 April 2025